|  |  | Unit Rate | £ |
| --- | --- | --- | --- |
| 3.1.10 | Dayworks Before Practical Completion |  |  |
| (a) | Craftsmen hours: 8h |  |  |
| (b) | Labourer’s hours: 8h |  |  |
| (c) | Include a provisional sum for materials to be used in daywork. | PS | £1,000.00 |
| (d) | Add for profit and overheads to be allowed on the net cost of materials used in dayworks. |  |  |
| (e) | Include a Provisional sum for plant, etc., to be used in dayworks. |  |  |
| (f) | Add profit and overheads. |  |  |
| 3.1.11 | Dayworks During Defects Liability Period |  |  |
| (a) | Craftsmen hours: 8h |  |  |
| (b) | Labourer’s hours: 8h |  |  |
| 3.1.12 | #### Contingencies |  |  |
|  | Include a Provisional sum for contingencies | PS | £5,000.00 |
| DAYWORKS & CONTINGENCIES TO COLLECTION |  |  | £ |

| # Item |  | ## Unit | Rate | Cost (£) |
| --- | --- | --- | --- | --- |
| 3.2 | # GENERAL ITEMS |  |  |  |
| 3.2.1 | Hours of Working Works to be undertaken only during normal working hours, unless otherwise agreed by C.A. (Monday – Saturday 8:00am – 5:30pm) | Item |  |  |
| 3.2.2 | Programme of Works The Contractor will be required to provide a PROGRAMME OF WORKS before works commence to ensure minimal disruption to the Client. | Item |  |  |
| 3.2.3 | Site Protection and Tidiness Contractor to allow for keeping the site clean tidy at all times, and for the disposal of all waste material both during and on completion of the works. | Item |  |  |
| 3.2.4 | Waste Disposal Allow for all skips, licences and rubbish disposal throughout the duration of the works. Skips to be located in positions agreed before the commencement of the works. | Item |  |  |
| 3.2.5 | Temporary Works The Contractor is to allow for all necessary shoring, formworks and their temporary works as required | Item |  |  |
| 3.2.6 | Access Equipment Include for all scaffolding, access towers and safety netting, etc. required to safely undertake the works. All standards and poles lower than 3m are to be provided with alternative red and white coloured tape. All ladders to lower lifts are to be removed and secured overnight. Full toe boards and safety netting to all lifts. | Item |  |  |
| 3.2.7 | Noise and Disturbance The use of noisy plant or machinery is to be kept to a minimum and site radios will not be allowed. Please note the site will continue to be in use by the client and it may be necessary for the contractor to stop work on occasions when activities are taking place. Where possible the client will provide a schedule of bookings when the date for the proposed works is agreed. | Item |  |  |
| 3.2.8 | Site Safety Include for all protective and other safety equipment such as barriers, safety signage, and a designated safe place to leave ladders and materials while the site is unattended. | Item |  |  |
| 3.2.9 | Site Enclosure The Contractor is to allow for providing a full enclosure to the site in herras fencing, and to agree a compound position with the C.A. | Item |  |  |
| 3.2.10 | Welfare Facilities Provide temporary latrines and welfare facilities for site staff if required. | Item |  |  |
| 3.2.11 | All works to be adequately weather protected at all times. | Item |  |  |
| 3.2.12 | Personal Protective Equipment All operatives should be properly trained for their duties, and provided with all necessary PPE. | Item |  |  |
| 3.2.13 | Leadwork All leadwork to be in accordance with LDA guidelines – which can be supplied on request. | Item |  |  |
| 3.2.14 | CDM Regulations The full extent of the CDM Regulations 2015 will apply. The Contractor shall be required to submit the usual construction plan and maintain site visitors’ procedures and records, and accident books, etc. ALL accidents are to be reported to the C.A. and Principal Designer immediately. | Item |  |  |
| 3.2.15 | Health and Safety File The Contractor is to provide a list of all products and materials used on site, retain all manufacturer’s literature and contact details, to be handed over to the Principal Designer throughout the project, including commissioning certificates for services, etc. The C.A. reserves the right to delay final payment or agreement of the final account until this is completed. | Item |  |  |
| 3.2.16 | Refurbishment & Demolition Survey A type II pre-refurbishment Asbestos survey of the property has not been provided. Allow to undertake a full asbestos refurbishment and demolition survey prior to undertaking any works. A copy of the report is to be provided to the C.A. prior to any works. | Item |  |  |
| 3.2.17 | The Contractor is to notify the C.A. or Principal Designer immediately if any suspected asbestos containing materials are identified. | Item |  |  |
| 3.2.18 | Contractor to allow for providing all temporary lighting and power as required to safely complete works. | Item |  |  |
| 3.2.19 | BWIC Allow for builders’ work in connection with Mechanical & Electrical Services installation. Include for forming and finishing all necessary ducts, chases. The provision of all access towers, access equipment, etc. | Item |  |  |
| 3.2.20 | Tenant Fixtures & Fittings Assume that the tenants will be responsible for moving any internal belongings, furniture and equipment. All moved items to be stored on site. Any damaged areas on moving of the items is to be made good. | Item |  |  |
| 3.3 | DEMOLITION & STRIPPING OUT |  |  |  |
| 3.3.1 | Dual Pitched Roof Any sheared, missing or defective fixings to the profiled metal roof sheets are to be removed and disposed of. | Item |  |  |
| 3.3.2 | Rainwater Goods Carefully remove and dispose of the existing uPVC and cast iron downpipes, gutters and associated fixings and brackets. | Item |  |  |
| 3.3.3 | Soffits, eaves and bargeboards Carefully remove and dispose of timber soffits, eaves and bargeboards, leaving backing timbers in place. | Item |  |  |
| 3.3.4 | Temporarily remove tenants CCTV cameras and wiring without causing any damage to the supply cables and set aside, ready for re-fixing upon completion. | Item |  |  |
| 3.4 | DUAL PITCHED ROOF |  |  |  |
| 3.4.1 | Fixings On removal of the defective fixings, supply and install matching galvanised or stainless steel fixings and ensure they are securely fitted to the existing profiled metal roof sheets. | Item |  |  |
| 3.4.2 | Cut Edge Corrosion There is cut edge corrosion to a number of the profiled roof sheets. Carefully, using an abrasive brush, prepare the surfaces by removing all loose and flaking metal and feather in any lifting edges to the roof sheets. Apply a surface and wet tolerant corrosion epoxy primer to the corroded areas and allow the primer to cure sufficiently before applying the silicone coating. | Item |  |  |
| 3.4.3 | Once the existing roof sheets have been primed, supply and install Liquasil Ltd Flexlap self-levelling silicone coating cut edge corrosion treatment, in a RAL colour to match the existing profiled metal roof sheet. |  |  |  |
| 3.4.3 Cont’d | As supplied by: Liquasil Ltd Unit 3 Radway Industrial Estate Radway Road Solihull West Midlands B90 4NR Phone – 0121 709 5352 Email – info@liquasil.com https://liquasil.com/cut-edge-corrosion-treatment/ It is required that the application of the silicone coating is undertaken as specified by Liquasil Ltd. 15 year product warranty and a service life (BBA Certification) backed warranty of 25 years is required. | Item |  |  |
| 3.4.4 | Any areas where the cut edge corrosion is particularly severe, it may be required that gaps are to be sealed to the overlapping sheets. If required, supply and install Liquasil HP Butyl Waterproofing Tape. Prior to its application, ensure all surfaces are dry and free from grease and the mid-laps have been primed and cured with surface and wet tolerant primer. Using a penny roller carefully apply the tape to the profiled metal roof sheets and ensure there are no gaps where water can ingress. |  |  |  |
|  | As supplied by: Liquasil Ltd Unit 3 Radway Industrial Estate Radway Road Solihull West Midlands B90 4NR Phone – 0121 709 5352 Email – info@liquasil.com https://liquasil.cdn.prismic.io/liquasil/e39f7ae0-bd9c-4d93-9566-8bf612d74513_Liquasil+HP+Butyl+Tape+.pdf It is required that the application of the waterproofing tape is undertaken as specified by Liquasil Ltd. |  |  |  |
| 3.4.5 | Liquid Applied Metal Roof Coating System Carefully clean and prepare the existing profiled metal roof sheets, removing any debris and stabilise all existing finishes. Apply a surface and wet tolerant corrosion epoxy primer to the corroded areas and allow the primer to cure sufficiently before applying the silicone coating. | Item |  |  |
| 3.4.6 | Supply and install Liquasil Ltd Metalseal metal roof coating system to the entire pitched roof as highlighted on the floor plan. Ensure two coats are provided and a smooth, even surface is provided, in a RAL colour that matches the existing profiled metal roof sheets and Liquasil Ltd Flexlap coating. As supplied by: Liquasil Ltd Unit 3 Radway Industrial Estate Radway Road Solihull West Midlands B90 4NR Phone – 0121 709 5352 Email – info@liquasil.com https://liquasil.com/metal-roof-coating/ It is required that the application of the metal roof coating system is undertaken as specified by Liquasil Ltd. | Item |  |  |
| 3.4.7 | Include for new flashing and upstands at the junction of the new valley gutter formations with the previously repaired valley details at the junction of the earlier repaired roof. | Item |  |  |
| 3.4.8 | Allow for water testing of junctions to ensure watertightness. | Item |  |  |
| 3.5 | uPVC WINDOWS |  |  |  |
| 3.5.1 | The existing uPVC windows are to be retained but may have to be temporarily removed and store before re-fitting. | Item |  |  |
| 3.5.2 | Temporarily unbolt and remove metal security grilles retaining and set aside grilles for re-use. | Item |  |  |
| 3.5.3 | Allow to carefully remove window glazing and uPVC window frames to allow removal of damaged uPVC window cills. | Item |  |  |
| 3.5.4 | Provide and fit new metal window cills in powder coated aluminium (Colour: white) with adequate overhanging drip detail sized to replace damaged uPVC cills. | Item |  |  |
| 3.5.5 | Re-fit existing uPVC window frames into original openings. | Item |  |  |
| 3.5.6 | Allow for re-fitting any double glazed panes set aside using new rubber glazing seals. | Item |  |  |
| 3.5.7 | Allow for 50% glass replacement due to breakages during removal or storage. | Item |  |  |
| 3.5.8 | Allow for new external mastic sealant around window heads, reveals and cills. | Item |  |  |
| 3.5.9 | Allow for making good internal wall surfaces, window boards and decorative repairs around internal areas of window frames and openings. | Item |  |  |
| 3.5.10 | Window Metal Security Grilles Following completion of window repairs, re-fit existing window metal grilles using new fixing bolts, set into resin, using original fixing positions in brickwork. Allow for new fixing plugs, if required. | Item |  |  |
| 3.6 | RAINWATER GOODS |  |  |  |
| 3.6.1 | Replace Rainwater Goods Carefully remove the existing rainwater goods; following completion of the replacement fascia and soffit details. | Item |  |  |
| 3.6.2 | Supply and install new 6inch Half Round Profile seamless gutter system and downpipes in the same locations, please refer to the Roof Plan drawing. The colour is to match the existing rainwater goods finish. All rainwater goods are to be fixed with stainless or galvanised steel fixings and brackets. |  |  |  |
| 3.6.2 Cont’d … | As supplied by (or similar to): Seamless Aluminium UK Unit 6b Solihull Building Trade Centre Richmond Road Solihull B92 7RN Call: +44 (0)121 765 4355 Email: info@seamlessaluminium.co.uk https://seamlessaluminium.co.uk/product/half-round-seamless-gutter/ Ensure all gutters have uniform fall to ensure run-off to the downpipes. Ensure all existing profiled roof sheets fall into the new gutter system preventing any ware running down the external elevations or over spilling the gutter system. | Item |  |  |
| 3.6.3 | Flush through all newly installed rainwater goods and ensure there are no blockages or leaks. | Item |  |  |
| 3.6.3 | Supply and install polypropylene downpipe filters to all downpipe, 6No in total. As supplied by (or similar to): Drainage Superstore Burrington Business Park Burrington Way Plymouth Devon PL5 3LX https://www.drainagesuperstore.co.uk/search?keywords=polyproplene Product Code: RO-LEAFGUARD-210-020-080-125-Z - Wallbarn | 6No |  |  |
| 3.7 | FASCIAS & SOFFITS |  |  |  |
| 3.7.1 | On removal of the existing rainwater goods, remove the timber fascia, soffit and bargeboard details. | Item |  |  |
| 3.7.2 | Provide and fit new 12mm WBP plywood fascias, soffits, bargeboard and end caps to match the original section sized of the removed items; to be fixed to existing timber backing bearers, rafter feet and battens. | Item |  |  |
| 3.7.3 | Allow for a Provisional sum of £2,000 for labour and materials to repair any unseen damage behind original fascia and soffit construction. | PS |  | £2,000.00 |
| 3.7.4 | Decorate the timber soffits and fascia with external quality timber solid wood stain, such as Ronseal Trade Woodstain. Decoration to be in accordance with BS EN 234:2019. To include following sequence as a minimum: - Cleaning and de-greasing in preparation. - Rub down surfaces to remove blemishes and provide key. - One base coat. - Rub down. - One top / finish coat. | Item |  |  |
| 3.7.5 | Include for re-fixing tenants CCTV cameras and wiring in original positions, including drilling the cable through the soffit and testing to ensure the CCTV is in working condition upon completion. | Item |  |  |
| 3.8 | MISCELLANEOUS ITEMS |  |  |  |
| 3.8.1 | Prepare and paint metal louvre grilles at high level on west gable in matching colours | Item |  |  |
| 3.8.1 | Metal Box Gutter above Main Entrance Clean out al silt and debris. | Item |  |  |
| 3.8.2 | Re-seal joints. | Item |  |  |
| 3.8.3 | Prepare and paint metal louvre grilles at high level on west gable in matching colours. | Item |  |  |
| 3.8.4 | Fire Escape Door Remove existing fire escape door and frame. | Item |  |  |
| 3.8.5 | Provide and fit new metal fire escape door and frame complete with new panic escape fire door ironmongery on the interior: As supplied by: Latham’s Stee Security www.lathamssteeldoors.co.uk 01384 220050 Ref: Security Fire Exit Door with Exidor 294 adjustable 2-point security exit push bar. | Item |  |  |
| 3.8.6 | Security Grilles around External Bulkhead Light Fittings Remove and dispose of existing security grilles around light fittings. | Item |  |  |
| 3.8.7 | Check that the light fittings work. | Item |  |  |
| 3.8.8 | Allow for the provisional sum of £2,500 for labour and materials to: - Re-lamp existing light fittings. - Provide replacement diffusers to existing fittings. - Provide and install new vandal resistant light fittings with opaque diffusers. | PS |  | £2,500.00 |
| 3.8.9 | Provide and install new bespoke anti-vandal grilles around repaired light fittings. | Item |  |  |
| 3.9 | CLEANING & HANDOVER |  |  |  |
| 3.9.1 | Upon completion of the works fully clean all areas, removing all contractors’ debris and litter, etc. | Item |  |  |
| 3.10 | DOCUMENTATION |  |  |  |
| 3.10.1 | Provide 2No O&M Manuals in hard copy printed format and 1No USB Stick. | Item |  |  |
| 3.10.2 | Provide the insurance backed guarantee for the re-roofing. | Item |  |  |

3.11	 CONTRACTOR'S OWN ADDITIONS

| Item |  | £ |
| --- | --- | --- |
|  | Items that the Contractor feels have been omitted from this Specification of Works but in his opinion are deemed necessary for inclusion to adequately and satisfactorily complete the works. Please specify and price below. Use an additional sheet if necessary. |  |
| 3.11.1 |  |  |
| 3.11.2 |  |  |
| 3.11.3 |  |  |
| 3.11.4 |  |  |
| 3.11.5 |  |  |
|  | ###### TOTAL TO COLLECTION | £ |

4.0	FINAL COLLECTION PAGE

|  |  | £ |
| --- | --- | --- |
| 1.0 | Preliminaries and General Conditions |  |
| 2.0 | Materials and Workmanship |  |
| 3.0 | Schedule of Works |  |
| 3.1 | Dayworks and Contingencies |  |
| 3.2 | General Items |  |
| 3.3 | Demolition & Strip-out |  |
| 3.4 | Dual Pitched Roof |  |
| 3.5 | uPVC Windows |  |
| 3.6 | Rainwater Goods |  |
| 3.7 | Fascias & Soffits |  |
| 3.8 | Miscellaneous Items |  |
| 3.9 | Cleaning & Handover |  |
| 3.10 | Documentation |  |
| 3.11 | Contractors’ Own Additions |  |
|  | TOTAL (excluding VAT) | £ |

5.0	FORM OF TENDER

Downside Community Centre

Suffolk Road

Dunstable

LU5 4ES

I/We................................................................................................................................................................................................................................................................. having visited the site and examined the scope of works, hereby Tender and undertake to execute and complete all the works required in accordance with the various Conditions of Contract and the Specification of Works, for the sum of:

in words.......................................................................................................................

in figures.......................................................................................................................

exclusive of VAT.

I/We can commence the works on ................................................ and require ....................................... weeks to complete the works.

This Tender remains open for consideration for 12 weeks from the date fixed for submission.

I/We understand that the Employer is not bound to accept the lowest or any Tender.

Dated this.................................................day of.................................................. 20

Name................................................................

Signature...........................................................

This completed 'Form of Tender' and priced Specification of Works are to be returned to Brasier Freeth, Welwyn Garden City office, no later than 12 (noon) on Monday 5th January 2026.